Test Title: Verify Kirchoff’s Voltage Law and Voltage Division

R4 Requirement

Purpose: The purpose of this test is to verify that Kirchoff’s Voltage Law and Voltage Division apply and work for all circuits.

Variables:

Independent Variables

Input Voltage

Control Variables

Resistance

Dependent Variables

Output Voltage

Uncontrollable Variables

Resistance Variance

Voltage Spiking

Measurement Error

Experimental Setup:

Figure 1: Experimental Setup

Procedure:

- Build a circuit with 3 resistors with the following nominal values:

  - 200k

  - 100k

  - 1k

- Wire the resistors in series such that the first resistor is 1k, the second resistor is 100k, and the third resistor is 200k.

- Wire W1 input to the start of the first resistor. Wire GND to the end of the third resistor.

- Wire 1+ and 1- to both ends of the third resistor to measure it’s voltage drop. Make sure 1- is wired to the same node as GND.

- Set an arbitrary test voltage in for W1 in Waveforms. Record the output voltage.

- Repeat two more times.

Sample Calculations:

V1 = (200k200k+100k+1k)1V=0.664V

V2 = (200k200k+100k+1k)3V=3.322V

V3 = (200k200k+100k+1k)=1.993V

R5 Requirement

Equipment:

- 3 Resistors

  - 200k

  - 100k

  - 1k

- Analog Discovery 3 board

- Male-to-male dupont connectors

Quantitative Results:

|  | Vin | V1 Calculated | V1 Measured (Tr 1) | V1 Measured (Tr 2) |
| --- | --- | --- | --- | --- |
| Input 1 | 1V | 0.644V | 636 mV | 632 mV |
| Input 2 | 3V | 3.322V | 1.8851 V | 1.8756 V |
| Input 3 | 5V | 1.993V | 3.1332 V | 3.1429 V |

Pictures:

Figure 2: 1 Volt Input Results

Figure 3: 3 Volt Input Results

Figure 4: 5 Volt Input Results

R6 Requirement

      - Observations and Conclusions:

- Observations:

  - The measured voltages across the 200 kΩ resistor did not match calculations but were close, within a few hundred mV.

  - At lower input voltages (1 V), the measured and calculated values were very close (less than 2% difference).

  - At higher input voltages (3V and 5V), larger gaps were displayed between calculated and measured values. The 3V test showed the largest gap, while the 5V test was near the middle.

- Discussion of Results:

  - The experiment supports Kirchhoff’s Voltage Law and Voltage Division, since the resistor voltages add up consistently to the applied input voltage.

  - The larger gaps between calculated and measured output voltages at higher input voltages could mean either measurement limitations, resistor tolerances, or noise.

- Possible Sources of Error:

  - Resistor tolerances: The resistors may not have been exact, leading to deviations from calculated values.

  - Measurements: The Analog Discovery 3 may have some small errors in measurement, especially at higher voltage, since it is a low voltage device with only a 5V power supply

- Conclusions:

  - The tests validates that KVL is true.

  - Voltage Division was observed, though real-world error prevented exact matching with the calculations.

- Potential Improvements:

  - Use resistors with tighter tolerances (like 0.1% instead of 1–5%).

  - Double-check the calculated voltage values before comparing to the measured data from the Discovery 3.

  - Use additional measuring equipment (a lab multimeter with a dedicated power supply) to verify results.

Conducted by: Corbin Hibler

Date test conducted: 2025-09-04